DayLab Networking Fundamentals

Hands-On Practice Guide for Network Engineers

Topics: Device Configuration & VLAN Fundamentals

Last Updated: April 2026

# Introduction

This hands-on guide provides practical lab exercises for foundational network engineering skills. Each day focuses on essential concepts that form the backbone of enterprise network administration.

Whether you&#x2019;re preparing for industry certifications or enhancing your operational skills, these labs emphasize real-world scenarios and business implications.

# Day 1: Basic Device Configuration

## Learning Objectives

- Configure fundamental switch/router parameters for operational readiness

- Implement security best practices through password policies and access control

- Document devices with system identity information for audit and management

- Persist configuration changes across device restarts

## Business Context

Proper device configuration is critical for network security, compliance, and operational efficiency. Misconfigurations lead to:

- Security breaches (weak passwords, default credentials)

- Lost configurations after power loss (cost: service downtime, potential compliance violations)

- Management issues (inability to identify devices, support escalations)

- Compliance failures (SOX, PCI-DSS require proper authentication and audit trails)

## Key Concepts

### 1. Hostname Configuration

The hostname is the device&#x2019;s logical identifier in the network. It should follow a naming convention that reflects location, function, and purpose.

Naming Convention Example SW-CORE-01 (Switch, Core, Unit 01) RTR-EDGE-NYC-02 (Router, Edge, NYC datacenter, Unit 02)

Business benefit:

- Quick identification of device role and location

- Simplified management and troubleshooting

- Clear asset inventory tracking

### 2. Password Configuration & Security

Cisco IOS devices support multiple authentication levels:

- User EXEC (Level 1): Limited commands, view-only operations

- Privileged EXEC (Level 15): Full device configuration access

- Enable Password: Legacy method (plaintext, not recommended)

- Enable Secret: Encrypted password (use MD5, SHA256, or SCRYPT)

- Line Passwords: Control access to Console, VTY (Telnet/SSH), and Aux ports

Security Best Practice Always use &#x201C;enable secret&#x201D; (encrypted) not &#x201C;enable password&#x201D; (plaintext) Pre-shared keys (PSK) in production: minimum 12+ characters, mixed case, numbers, symbols Console access: physical security + strong authentication = defense in depth

### 3. MOTD (Message of the Day)

Legal disclaimer banner displayed upon login. Two types:

- Banner MOTD: Shown before authentication (legal warning &#x2014; critical for compliance)

- Banner LOGIN: Shown after authentication

Business relevance: Legal protection. Courts require clear notice that access is authorized only for legitimate business purposes.

Compliance Requirement MOTD example: ***AUTHORIZED ACCESS ONLY*** This system is for authorized use only. Unauthorized access is a violation of law. All activity is monitored and logged. This banner deters casual access and provides legal standing if unauthorized access occurs.

### 4. Configuration Persistence (Save Config)

Cisco devices have two configuration storage areas:

- Running Config (RAM): Active configuration, lost on reboot

- Startup Config (NVRAM/Flash): Loaded on boot, persists across power failures

Business impact: Any configuration not saved is lost. Data center outages due to unsaved configs = expensive downtime.

Critical Workflow Enter configuration commands → verify with show commands test the changes in safe environment → save to NVRAM Command: copy running-config startup-config (or write memory) This is non-negotiable in production environments.

### 5. Show Commands for Verification

Essential verification commands:

| Command | Purpose |
| --- | --- |
| show version | Device model, IOS version, uptime, system resources |
| show running-config | Active configuration in RAM |
| show startup-config | Configuration that loads on boot |
| show interfaces | Port status, IP addresses, traffic statistics |
| show ip interface brief | Quick port status summary |
| show hostname | Current device name |
| show users | Active management sessions |

## Hands-On Lab Exercise

Estimated Time: 45 minutes | Equipment: 1 switch/router, console cable, terminal emulator

### Lab Scenario

You&#x2019;re deploying a new access switch for the corporate office. The equipment has shipped with factory defaults. You must configure it for production readiness: device identity, security controls, and configuration backup.

### Tasks

Task 1: Access the Device - Connect via console cable (DB-9 serial or USB) - Open terminal emulator (Putty, SecureCRT, or screen) - Baud rate: 9600 bps, 8 data bits, 1 stop bit, no parity - Power on the device (or reset with ROMMON if needed) - Expect to see boot messages and login prompt

Task 2: Enter Configuration Mode - At User EXEC prompt (>), enter: enable - At Privilege EXEC prompt (#), enter: configure terminal - You are now in Global Configuration mode

Task 3: Set Hostname - Command: hostname SW-CORP-ACC-01 - Verify: show running-config ¦ include hostname - Observation: Prompt changes to SW-CORP-ACC-01#

Task 4: Configure Enable Secret (Encrypted Password) - Command: enable secret 5 MySecureP@ss123 - Note: Type 5 indicates MD5 encryption (legacy, IOS handles it) - Modern devices support: enable secret 9 password (SCRYPT, recommended) - Verify: show running-config ¦ include enable - Look for the encrypted hash in output (not plaintext)

Task 5: Configure Console Access Password - Command: line console 0 - Subcommand: password ConsoleP@ss123 - Subcommand: login - Subcommand: exit - Verify: show running-config ¦ include line console -A 3 - Test: Disconnect and reconnect console, should be prompted for password

Task 6: Configure VTY (Telnet/SSH) Access Password - Command: line vty 0 4 - Subcommand: password VTYAccess#2024 - Subcommand: login - Subcommand: exit - Verify: show running-config ¦ include line vty -A 3 - Note: Best practice is to use SSH, not Telnet (covered in advanced labs)

Task 7: Configure MOTD Banner - Command: banner motd # (# is a delimiter) - Type the message, then # on a new line - Example message: - ***AUTHORIZED PERSONNEL ONLY*** - This system is for authorized corporate use. - Unauthorized access is prohibited by law. - All activity is monitored and logged. - # - Verify: show banner motd

Task 8: Verify All Configurations - show running-config (full config review) - show version (device details) - show ip interface brief (interface status) - Take screenshot of output for documentation

Task 9: Save Configuration to Startup - Command: exit (return to Privilege EXEC) - Command: copy running-config startup-config - Respond: [OK] - Verify: show startup-config - Alternative: write memory or copy run start (abbreviated forms)

Task 10: Test Configuration Persistence - Reload device: reload (confirm with 'y') - Wait for boot sequence (1-2 minutes) - When prompted, enter enable password (you set this in Task 4) - Enter Privilege EXEC - Verify: show running-config ¦ include hostname - Confirm all settings persisted: hostname, passwords, banners

## Troubleshooting Guide

| Password Locked Out | Recover via ROMMON break (Ctrl+Break) and password reset procedure. In production, have a documented recovery process. |
| --- | --- |

| Configuration Lost After Reboot | Likely reason: did not save running-config to startup-config. Always execute copy running-config startup-config before reload. |
| --- | --- |

| Console Not Responding | Check physical cable, terminal settings (9600, 8N1), and device power. Try different console port if available. |
| --- | --- |

| Cannot See MOTD | MOTD appears only on new connections. Disconnect and reconnect (or use new terminal session). |
| --- | --- |

# Day 2: Switching Basics & VLANs

## Learning Objectives

- Understand VLAN concepts and design rationale

- Create and configure VLANs on managed switches

- Assign ports to VLANs and configure trunking

- Verify VLAN membership and inter-VLAN communication

## Business Context

VLANs are fundamental to enterprise network design. They provide:

- Network segmentation and security isolation

- Flexibility in physical network changes without rewiring

- Broadcast domain management (reduced broadcast storms)

- Compliance with regulatory requirements (separate data, voice, guest networks)

- Cost savings: physical segmentation replaced by logical configuration

## Key Concepts

### 1. What is a VLAN?

A Virtual LAN (VLAN) is a logical segmentation of a physical switch into multiple independent broadcast domains. Each VLAN acts as a separate network.

Physical vs. Logical Physical network: 24-port switch = 1 broadcast domain With VLANs: Same switch = multiple broadcast domains Example: Port 1-8 (VLAN 10), Port 9-16 (VLAN 20), Port 17-24 (VLAN 30) Devices in VLAN 10 do NOT see broadcast traffic from VLAN 20 or 30.

### 2. VLAN IDs and Ranges

VLAN IDs are numeric identifiers from 1-4094:

- VLAN 1: Default VLAN (cannot be deleted, used for management by default)

- VLANs 2-1005: Normal range (stored in VLAN.dat on switch)

- VLANs 1006-4094: Extended range (dynamic, for provider environments)

VLAN Naming Convention VLAN 10: CORP-USERS (corporate end-users) VLAN 20: VOICE (IP phones) VLAN 30: DATA-CENTER (servers, backend) VLAN 99: MANAGEMENT (device management) VLAN 999: GUEST-WIFI (guest network, isolated) Convention = clarity = easier management

### 3. Access Ports vs. Trunk Ports

Two types of ports with different purposes:

| Port Type | Details |
| --- | --- |
| Access Port | Belongs to ONE VLAN End devices (PCs, phones, printers) Untagged traffic Typical: 90% of switch ports |
| Trunk Port | Carries MULTIPLE VLANs Switch-to-switch connections Tagged traffic (802.1Q tags) Typical: uplink ports to distribution/core |
| Native VLAN on Trunk | Untagged traffic on a trunk Default: VLAN 1 (should be changed for security) Used for management in older configs |

Tagged vs. Untagged:

- Tagged: 802.1Q header added (4 bytes) with VLAN ID

- Untagged: No header, switch assumes port&#x2019;s configured VLAN

Why Separate Port Types? Devices (PCs) don&#x2019;t understand VLAN tags. They send/receive frames with no tag. Access port removes tags from frames before sending to device. Switches understand tags and need them to route frames to correct VLAN. Trunk port keeps tags on frames for switch-to-switch communication.

### 4. Inter-VLAN Communication

By design, VLANs are isolated. VLAN 10 cannot communicate with VLAN 20 directly. To enable inter-VLAN routing:

- Layer 3 switch (router built-in, typical in modern networks)

- External router with trunk connection to switch

- Each VLAN needs an IP address (SVI: Switched Virtual Interface)

This is covered in advanced labs. For now, understand: VLANs isolate, Layer 3 devices enable communication between them.

## Hands-On Lab Exercise

Estimated Time: 60 minutes | Equipment: 1 managed switch (Catalyst or equivalent), console access

### Lab Scenario

A growing company has moved to a new office. Network requirements:

- VLAN 10: Corporate Users (Ports 1-8)

- VLAN 20: Voice (IP Phones) (Ports 9-12)

- VLAN 30: Data Center / Servers (Ports 13-16)

- VLAN 99: Management (for admin access)

- Uplink: Port 24 (trunk to distribution switch)

Your task: Configure the switch to implement this design.

Task 1: Review Current VLAN Configuration - Ensure you&#x2019;re in Privilege EXEC mode (#) - Command: show vlan brief - Observe: Only VLAN 1 exists (default) - Command: show vlan name * - Observation: All 4094 VLAN slots exist but are unused

Task 2: Create VLAN 10 (Corporate Users) - Enter Global Config: configure terminal - Command: vlan 10 - Subcommand: name CORP-USERS - Subcommand: exit - Verify: show vlan id 10

Task 3: Create VLAN 20 (Voice) - Command: vlan 20 - Subcommand: name VOICE - Subcommand: exit

Task 4: Create VLAN 30 (Data Center) - Command: vlan 30 - Subcommand: name DATA-CENTER - Subcommand: exit

Task 5: Create VLAN 99 (Management) - Command: vlan 99 - Subcommand: name MANAGEMENT - Subcommand: exit - Verify all: show vlan brief (should show VLANs 1, 10, 20, 30, 99)

Task 6: Configure Port 1 as Access Port in VLAN 10 - Command: interface fastEthernet 0/1 (or GigabitEthernet 0/1 for modern switches) - Subcommand: switchport mode access - Subcommand: switchport access vlan 10 - Subcommand: description CLIENT-PC-001 - Subcommand: no shutdown (if port is down) - Subcommand: exit - Verify: show interface fastEthernet 0/1 switchport

Task 7: Configure Ports 2-8 as Access Ports in VLAN 10 - Command: interface range fastEthernet 0/2 - 8 - Subcommand: switchport mode access - Subcommand: switchport access vlan 10 - Subcommand: no shutdown - Subcommand: exit - Verify: show vlan id 10 (should show ports 1-8 assigned)

Task 8: Configure Ports 9-12 as Access Ports in VLAN 20 (Voice) - Command: interface range fastEthernet 0/9 - 12 - Subcommand: switchport mode access - Subcommand: switchport access vlan 20 - Subcommand: description VOICE-PHONES - Subcommand: exit - Verify: show vlan id 20

Task 9: Configure Ports 13-16 as Access Ports in VLAN 30 (Data Center) - Command: interface range fastEthernet 0/13 - 16 - Subcommand: switchport mode access - Subcommand: switchport access vlan 30 - Subcommand: description DATA-CENTER-SERVERS - Subcommand: exit - Verify: show vlan id 30

Task 10: Configure Port 24 as Trunk Port - Command: interface fastEthernet 0/24 (or appropriate uplink port) - Subcommand: switchport mode trunk - Subcommand: switchport trunk allowed vlan 1,10,20,30,99 - Subcommand: switchport trunk native vlan 99 - Subcommand: description UPLINK-TO-DISTRIBUTION - Subcommand: exit - Verify: show interface fastEthernet 0/24 switchport - Look for mode: trunk, encapsulation: 802.1q, allowed VLANs

Task 11: Configure Management VLAN (VLAN 99) - Command: interface vlan 99 - Subcommand: ip address 192.168.99.10 255.255.255.0 - Subcommand: description MANAGEMENT - Subcommand: no shutdown - Subcommand: exit - Note: This SVI (Switched Virtual Interface) allows management access via SSH/Telnet

Task 12: Verify All VLAN Assignments - show vlan brief (comprehensive view of all VLANs and ports) - show vlan id 10 - show vlan id 20 - show vlan id 30 - show interface fastEthernet 0/1 switchport - show interface fastEthernet 0/24 switchport - show interface vlan 99 - Document output for your records

Task 13: Verify VLAN Membership via Mac Address Table - Simulate a device on Port 1 (VLAN 10): ping 192.168.10.1 (or check ARP) - Command: show mac-address-table vlan 10 - Observation: Shows learned MAC addresses in VLAN 10 on Port 1 - Command: show mac-address-table dynamic - Observe entries across different VLANs

Task 14: Test Port Isolation Between VLANs - Key concept: Devices on different VLANs cannot communicate without routing - Practical test (requires external router or Layer 3 switch): - - Connect a PC to Port 1 (VLAN 10) - - Connect another PC to Port 9 (VLAN 20) - - Try to ping between them: should FAIL (VLANs are isolated) - - This demonstrates VLAN security at Layer 2

Task 15: Test Trunk Port - Connect another switch to Port 24 (trunk port) - Verify frames tagged with VLAN IDs are passing - Advanced verification: debug or port analyzer showing 802.1Q headers - Confirm all allowed VLANs (1,10,20,30,99) pass through trunk

Task 16: Save Configuration - Command: exit (back to Privilege EXEC) - Command: copy running-config startup-config - Verify: show startup-config ¦ include vlan - Device should boot with all VLANs and port assignments intact

## Detailed Explanation of Key Tasks

### Why Access Ports?

End devices (PCs, phones, printers) do not understand VLAN tags. They send and receive standard Ethernet frames without 802.1Q headers. When you set a port to &#x201C;switchport mode access,&#x201D; the switch:

- Strips VLAN tags from frames sent TO the device

- Automatically tags frames FROM the device with the port&#x2019;s VLAN ID

- Isolates the device to its assigned VLAN

Result: A PC plugged into Port 1 (VLAN 10) can only communicate with other devices in VLAN 10.

### Why Trunk Ports?

Switches need to exchange VLAN information across multiple VLANs. A single link between switches could handle dozens of VLANs via tagging. When you set a port to &#x201C;switchport mode trunk,&#x201D:

- Port KEEPS 802.1Q VLAN tags on all frames

- You specify which VLANs are allowed (switchport trunk allowed vlan)

- Native VLAN = untagged traffic (typically VLAN 1, but should be changed to unused VLAN for security)

Result: A single uplink can carry frames from VLAN 10, 20, 30 all at once, differentiated by VLAN ID in the tag.

### Native VLAN on Trunk (Why Change It?)

Native VLAN = untagged traffic on a trunk port. Default is VLAN 1. Security risk:

- An attacker can inject untagged frames that land in VLAN 1

- VLAN 1 often carries management traffic

- Recommendation: Set native VLAN to an unused VLAN (e.g., 999) to isolate untagged traffic

In the lab, we used VLAN 99 as native. In production, use a dedicated &#x201C;black hole&#x201D; VLAN.

### VLAN Membership Verification

Three ways to verify VLAN assignments:

- show vlan brief: Displays all VLANs and member ports

- show vlan id X: Details for specific VLAN (interfaces, names, status)

- show interface Y switchport: Details for specific port (access/trunk mode, native VLAN, allowed VLANs)

Always use all three to confirm configuration matches design.

## Troubleshooting Guide

| Device Can't Access Network (Connected to VLAN but No IP) | 1. Port is in access mode but VLAN not created 2. VLAN created but port not assigned 3. Device obtains DHCP IP from wrong subnet Fix: Verify port membership (show interface X switchport), VLAN exists (show vlan id Y), and device can reach DHCP server. |
| --- | --- |

| Trunk Port Shows 'Trunking Encapsulation Desirable' Instead of 'Up' | Likely cause: Encapsulation mismatch between switches (one ISL, one 802.1Q) Fix: Explicitly set encapsulation: switchport trunk encapsulation dot1q |
| --- | --- |

| Port Shows as 'err-disabled' or 'notconnect' | Port may be shut down or have a hardware error Fix: no shutdown, or reload port if persistent |
| --- | --- |

| Devices on Same VLAN Can't Ping Each Other | 1. Port mode is wrong (not access) 2. VLAN not created 3. Native VLAN issue on trunk (less common) 4. Spanning Tree blocking port (advanced) Basic fix: Verify access port mode and VLAN membership. |
| --- | --- |

## Key Takeaways

- VLANs are logical segmentation of physical switches

- Access ports: For end devices (untagged)

- Trunk ports: For switch-to-switch (tagged, multiple VLANs)

- Verify with: show vlan brief, show vlan id X, show interface Y switchport

- Always save config: copy running-config startup-config

- Plan naming conventions for scalability

- Change native VLAN on trunks for security (not VLAN 1)

# Next Steps & Advanced Topics

After mastering Days 1-2, you&#x2019;re ready for:

- Day 3: IP Routing & Inter-VLAN Routing (Layer 3 switches, static routes)

- Day 4: Spanning Tree Protocol (preventing loops, understanding convergence)

- Day 5: SSH & Remote Access Security (replacing Telnet)

- Day 6: DHCP & DNS Configuration

- Day 7: Etherchannel & Port Aggregation (redundancy & bandwidth)

- Day 8: ACLs (Access Control Lists) for security

- Day 9: Monitoring & Troubleshooting (SNMP, syslog, NetFlow)

- Day 10: Security Best Practices & Compliance (passwords, logging, updates)