CCNA Industry-Level Must-Do Labs

Complete Port Numbers & Protocol Definitions Guide

# 1. ROUTING PROTOCOLS

## OSPF (Open Shortest Path First)

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| OSPF | IP Protocol 89 | Routing Protocol | Multi-router connectivity & dynamic routing |
| OSPF Hello | 224.0.0.5:89 | Multicast | Neighbor discovery & keepalive |
| OSPF DR/BDR Election | 224.0.0.6:89 | Multicast | Designated Router election |

## Static Routing & Default Routes

Static Route: Manually configured, requires explicit configuration for each destination network

Default Route (0.0.0.0/0): Catch-all route when no other route matches; used for gateway-of-last-resort

# 2. SWITCHING & VLAN PROTOCOLS

## VLAN (Virtual Local Area Network)

802.1Q Trunking: Encapsulates VLAN ID in frame headers; supports up to 4094 VLANs

Native VLAN: Untagged traffic on trunk; default VLAN 1 (mismatch = VLAN hopping risk)

## STP (Spanning Tree Protocol)

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| STP BPDU | 01:80:C2:00:00:00 | MAC Address | Bridge Protocol Data Unit (multicast) |
| Hello Timer | 2 seconds | Default | Bridge sends hello packets |
| Forward Delay | 15 seconds | Default | Listening/Learning state transition |
| Max Age | 20 seconds | Default | When BPDU expires/root lost |

Root Bridge Election: Lowest Bridge ID (Priority 0-61440 + MAC); recalculation on network change

RSTP (Rapid STP): IEEE 802.1w; convergence in seconds instead of minutes; backward compatible

## EtherChannel (Port Aggregation)

LACP (Link Aggregation Control Protocol): IEEE 802.3ad; dynamic negotiation of multiple links as one logical link

PAgP (Port Aggregation Protocol): Cisco proprietary; less flexible than LACP

Load Balancing: Source/Destination IP, Source/Destination MAC, or Layer 4 ports

# 3. SECURITY PROTOCOLS

## ACL (Access Control Lists)

Standard ACL: Filters by source IP only (ACL 1-99, 1300-1999)

Extended ACL: Filters by source/destination IP, protocol, port (ACL 100-199, 2000-2699)

Named ACL: Readable names; easier to edit (named-acl commands)

## SSH (Secure Shell)

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| SSH | TCP 22 | Management | Encrypted remote login (replaces Telnet) |
| Telnet | TCP 23 | Management | Unencrypted; avoid in production |
| SSH Key Exchange | TCP 22 | Cryptography | RSA 1024/2048-bit or ECDSA |

## Port Security

Dynamic MAC Learning: Learns up to max allowed MACs per port; violates on excess

Violation Modes: Shutdown (err-disable), Restrict, Protect

Sticky MAC: Dynamically learn & retain MACs; persists across reboot with 'sticky' save

# 4. NETWORK SERVICES PROTOCOLS

## DHCP (Dynamic Host Configuration Protocol)

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| DHCP Server | UDP 67 | Server Port | Receives client DHCP requests |
| DHCP Client | UDP 68 | Client Port | Sends DHCP requests |
| DHCP Discover | 255.255.255.255:67 | Broadcast | Client broadcasts to find servers |
| DHCP Offer | UDP 68 | Unicast/Broadcast | Server offers IP configuration |
| DHCP Request | 255.255.255.255:67 | Broadcast | Client requests offered config |
| DHCP ACK | UDP 68 | Unicast | Server confirms IP lease |

DHCP Lease: Default 24 hours; renewals at 50% (12hr) and 87.5% (21hr) of lease time

DHCP Relay (ip helper-address): Forwards broadcasts to DHCP server on different subnet; converts to unicast

## NAT (Network Address Translation)

Static NAT: 1:1 permanent mapping (e.g., internal 10.1.1.5 = external 203.0.113.5)

Dynamic NAT: 1:1 temporary mapping; requires pool of public IPs

PAT (Port Address Translation/Overload): Many-to-1; uses source port to differentiate internal hosts (e.g., 203.0.113.1:1000-65535)

# 5. ROUTER REDUNDANCY

## HSRP (Hot Standby Router Protocol)

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| HSRP | UDP 1985 | Redundancy | Active/Standby router coordination |
| HSRP Multicast | 224.0.0.2:1985 | Multicast | Heartbeat between HSRP routers |
| HSRP Virtual IP | VIP (e.g., 10.1.1.254) | Default Gateway | Clients point to this, switches between active/standby |

Active Router: Highest priority (0-255) or highest IP; forwards all traffic

Standby Router: Second highest priority; takes over if active fails

Preempt: If enabled, higher priority router forces takeover (default: no preemption)

# 6. SWITCH SECURITY FEATURES

## PortFast

Purpose: Skips STP listening/learning states (30 seconds) on access ports

Use Case: End devices (hosts, printers); never on trunk ports

## BPDU Guard

Purpose: Disables (err-disable) port if unsolicited BPDU received

Benefit: Prevents accidental STP loops from rogue switches

# 7. TRUNK CONFIGURATION

## 802.1Q Trunk

Encapsulation: 4-byte header with VLAN ID (12 bits); supports VLANs 1-4094

Native VLAN: Default VLAN 1; untagged on trunk; mismatch = DTP issue

DTP (Dynamic Trunking Protocol): Cisco proprietary; negotiates trunk mode (desirable/auto) or disables with 'nonegotiate'

# 8. INTER-VLAN ROUTING

## Router-on-a-Stick (ROAS)

Configuration: Single physical interface with subinterfaces; one per VLAN (e.g., Gi0/1.10, Gi0/1.20)

Encapsulation: Each subinterface tagged with 802.1Q VLAN ID

Routing: Router assigns IP to each subinterface; inter-VLAN traffic routed between subinterfaces

# 9. CRITICAL PORT NUMBERS QUICK REFERENCE

## Management & Monitoring

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| SSH | TCP 22 | Management | Secure device access |
| Telnet | TCP 23 | Management | Unencrypted access (legacy) |
| SNMP | UDP 161 | Monitoring | Network device monitoring |
| SNMP Trap | UDP 162 | Notification | Device alerts to manager |
| Syslog | UDP 514 | Logging | Centralized log collection |

## Application Layer

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| DNS | UDP 53 (TCP 53) | Query/Zone Transfer | Name resolution |
| HTTP | TCP 80 | Web | Unencrypted web traffic |
| HTTPS | TCP 443 | Web | Encrypted web traffic (SSL/TLS) |
| SMTP | TCP 25 | Email | Mail server to server |
| SMTP Submission | TCP 587 | Email | Client to mail server (TLS) |

## Routing Protocols

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| RIP | UDP 520 | Routing | Distance-vector routing (legacy) |
| OSPF | IP 89 | Routing | Link-state routing; multicast 224.0.0.5/6 |
| EIGRP | IP 88 | Routing | Cisco proprietary; multicast 224.0.0.10 |
| BGP | TCP 179 | Routing | External gateway protocol |

## DHCP & BOOTP

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| DHCP Server | UDP 67 | DHCP | Receive client requests |
| DHCP Client | UDP 68 | DHCP | Send requests & receive offers |
| BOOTP Server | UDP 67 | Legacy | Bootstrap protocol (precedes DHCP) |
| BOOTP Client | UDP 68 | Legacy | Client requests via BOOTP |

# 10. TROUBLESHOOTING COMMAND QUICK REFERENCE

## OSPF Troubleshooting

- show ip ospf neighbor — Verify neighbor adjacency

- show ip ospf interface — Check interface OSPF status

- show ip ospf database — View OSPF LSA database

- debug ip ospf adj — Verbose neighbor adjacency output

## ACL Troubleshooting

- show ip access-lists — View all ACLs and hit counts

- show ip access-lists 100 — View specific ACL details

- show run | include access-list — Show running ACL config

- clear access-list counters — Reset ACL hit counts

## NAT Troubleshooting

- show ip nat translations — Active NAT translations

- show ip nat statistics — Hits, misses, and table size

- debug ip nat — Packet-level NAT translation details

## VLAN & Trunk Troubleshooting

- show vlan brief — List all VLANs and ports

- show interfaces trunk — Verify trunk status & native VLAN

- show dtp interface Gi0/1 — DTP mode & status

- show spanning-tree — STP status & root bridge info

# 11. LAB PREREQUISITES & SETUP

## Hardware/Software Requirements

- Minimum 2 Routers, 2-3 Switches (Cisco 2800/3800 series recommended)

- IOS 12.4 or higher; latest is 15.x

- Console cables, crossover/straight-through cables

- Simulator: Cisco Packet Tracer or GNS3 (alternative to physical lab)

## Critical Configurations (All Labs)

- Enable secret password (encrypted enable password)

- Configure SSH (RSA 1024-2048 key generation)

- Set hostname & IP addressing (plan subnetting first)

- Configure NTP (time synchronization across devices)

# 12. KEY METRICS & THRESHOLDS

## OSPF Convergence Times

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| Hello Interval | 10 sec (LAN), 30 sec (WAN) | Default | Neighbor keepalive |
| Dead Interval | 40 sec (LAN), 120 sec (WAN) | Default | When neighbor considered down (4x hello) |
| Convergence Time | <15 seconds typical | Target | Time to detect failure & recalculate routes |

## HSRP Timers

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| Hello Timer | 3 seconds | Default | Multicast heartbeat interval |
| Hold Timer | 10 seconds | Default | Failure detection (must be > hello) |
| Failover Time | ~10-15 sec | Target | Time to switch from Active to Standby |

## STP Convergence

| Protocol | Port(s) | Type | Purpose |
| --- | --- | --- | --- |
| Forward Delay | 15 sec | Default | Transition through Listening/Learning |
| Max Age | 20 sec | Default | BPDU timeout; detection of root loss |
| Total Convergence | 30-50 sec | Typical | Initial STP convergence (2x forward delay) |

# 13. INDUSTRY BEST PRACTICES

## Routing Security

- OSPF authentication (MD5 or SHA) on all neighbor links

- Distribute-list ACLs to filter routing advertisements

- Default-information originate with caution

## Switching Security

- Disable DTP on trunk ports (no negotiate)

- Set all access ports to explicit VLAN (avoid default VLAN 1)

- Enable port security on access switches (limit 1-2 MACs per port)

- Enable BPDU Guard on all PortFast-enabled ports

## IP Services

- DHCP: Use DHCP Snooping to prevent rogue DHCP servers

- NAT: ACLs define inside/outside IPs; PAT for many-to-one translation

- HSRP: Use authentication & track object to verify active router health

## Management

- Always SSH, never Telnet (deprecated in IOS 15.1+)

- Enable exec-timeout to prevent idle session exposure

- Log all configuration changes via syslog; audit trail for compliance